# Pre Lab 5: Measuring a Refractive Index

# The Refractive Index

The refractive index is a dimensionless value that describes how much the speed of light is reduced in a material compared to a vacuum. It is defined by the equation

n=cv

Where c is the speed of light in a vacuum, and v is the speed of light in a material. This also means that the value of n≥1.

It is used by Snell’s Law, n1sinθ1=n2sinθ2, to calculate the angle of refraction at an interface between two media, as well as the critical angle for total internal reflection.

This property doesn’t just apply to visible light, but also relevant to electromagnetic signals in general.

# Solid Polyethylene

The refractive index of solid polyethylene is n≈1.5

# The RG-58 Coaxial Cable

RG-58 is a common 50-ohm coaxial cable with four concentric layers from inside out:

- Center conductor: a solid or stranded copper wire (~0.9 mm diameter) that carries the signal.

- Dielectric insulator: solid polyethylene (PE) surrounding the center conductor.

- Outer conductor/shield: a braided copper mesh that acts as the return path and shields against electromagnetic interference.

- Outer jacket: a PVC coating for physical protection.

For signal transmission, an electromagnetic wave propagates through the dielectric between the inner and outer conductors, essentially a guided wave confined in that polyethylene layer. The signal travels slower than the speed of light in vacuum because of the dielectric. The ratio is exactly what the refractive index captures: v =cn.

For polyethylene with n ≈ 1.5, the signal travels at roughly c1.5≈2×108ms  , or about 66% the speed of light. This is often listed as the cable's "velocity factor" (~0.66 for RG-58).

# Digital Oscilloscope and Waveform Generator

## Function Generator (B&K Precision 4010A/4011A)

This generates repeatable electrical signals of known shape, frequency, and amplitude. From the datasheet, it can output sine, square, triangle, pulse, and ramp waveforms across a wide frequency range (up to 2 MHz for the 4010A, 5 MHz for the 4011A). It has a 50 Ω output impedance and can deliver up to 20 Vpp into an open circuit.

## Digital Oscilloscope (Tektronix TBS1000B)

This captures and displays voltage as a function of time. It digitizes the incoming signal using an analog-to-digital converter at a high sample rate, then displays the waveform on screen.

The function generator sends a pulse into the coaxial cable. The oscilloscope displays both the sent and received signals.